SBL

Terminology:

- stitched output is what you directly flash to SPI.

- OS container from GenCapsuleFirmware.”
Later firmware update → use OS container from GenCapsuleFirmware.

- SBL reads it at boot and applies updates to itself or other firmware regions.

- So it requires SBL running

- Contains: SBL, and keys

- Encapsulate firmware updates (SBL, microcode, ACPI tables, other firmware regions) in a single package.

- Include metadata: version, component identifiers, offsets, and update instructions.

- Enable authentication and integrity checks: signed container ensures that SBL only applies verified updates.

# Linux SBL→ Linux payload

https://slimbootloader.github.io/how-tos/boot-with-linux-payload.html

### Build SBL

cat linuxboot.patch

diff --git a/Platform/QemuBoardPkg/BoardConfig.py b/Platform/QemuBoardPkg/BoardConfig.py

index 61f6e156..b3b693ca 100644

--- a/Platform/QemuBoardPkg/BoardConfig.py

+++ b/Platform/QemuBoardPkg/BoardConfig.py

@@ -110,7 +110,7 @@ class Board(BaseBoard):

self.TEST_SIZE            = 0x00001000

self.SIIPFW_SIZE          = 0x00010000

-        self.EPAYLOAD_SIZE        = 0x0020D000

+        self.EPAYLOAD_SIZE        = 0x00500000

self.PAYLOAD_SIZE         = 0x00021000

self.CFGDATA_SIZE         = 0x00001000

self.KEYHASH_SIZE         = 0x00001000

@@ -131,7 +131,7 @@ class Board(BaseBoard):

self.TOP_SWAP_SIZE  = 0x020000

self.REDUNDANT_SIZE     = 0x080000

self.NON_VOLATILE_SIZE  = 0x001000

-            self.NON_REDUNDANT_SIZE = 0x2DF000

+            self.NON_REDUNDANT_SIZE = 0x56e000

self.SLIMBOOTLOADER_SIZE = (self.TOP_SWAP_SIZE + self.REDUNDANT_SIZE) * 2 + \

@@ -171,7 +171,7 @@ class Board(BaseBoard):

self.STAGE1_STACK_SIZE    = 0x00002000

self.STAGE1_DATA_SIZE     = 0x0000E000

-        self.LOADER_RSVD_MEM_SIZE = 0x00800000

+        self.LOADER_RSVD_MEM_SIZE = 0x00910000

self.CFG_DATABASE_SIZE    = self.CFGDATA_SIZE

# Add following to force to use a specific platform ID

diff --git a/Platform/QemuBoardPkg/CfgData/CfgDataExt_Brd1.dlt b/Platform/QemuBoardPkg/CfgData/CfgDataExt_Brd1.dlt

index 50a34116..3984a543 100644

--- a/Platform/QemuBoardPkg/CfgData/CfgDataExt_Brd1.dlt

+++ b/Platform/QemuBoardPkg/CfgData/CfgDataExt_Brd1.dlt

@@ -15,7 +15,7 @@ PLATFORMID_CFG_DATA.PlatformId | 1

PLAT_NAME_CFG_DATA.PlatformName            | 'QEMU_01'

-GEN_CFG_DATA.PayloadId                     | 'AUTO'

+GEN_CFG_DATA.PayloadId                     | 'LINX'

GEN_CFG_DATA.CurrentBoot                   | 31

!include CfgDataExt_Inc.dlt

diff --git a/Platform/QemuBoardPkg/CfgData/CfgDataExt_QSP.dlt b/Platform/QemuBoardPkg/CfgData/CfgDataExt_QSP.dlt

index 31f5664f..c2545963 100644

--- a/Platform/QemuBoardPkg/CfgData/CfgDataExt_QSP.dlt

+++ b/Platform/QemuBoardPkg/CfgData/CfgDataExt_QSP.dlt

@@ -15,7 +15,7 @@ PLATFORMID_CFG_DATA.PlatformId | 2

PLAT_NAME_CFG_DATA.PlatformName            | 'QSP'

-GEN_CFG_DATA.PayloadId                     | 'AUTO'

+GEN_CFG_DATA.PayloadId                     | 'LINX'

GEN_CFG_DATA.CurrentBoot                   | 31

!include CfgDataExt_Inc.dlt

ranshal@D-P-RANSHAL-L-RF-L-RF:~/projects/slimbootloader$

python BuildLoader.py build <platform> -p "dummy.txt:OSLD;bzImage:LINX;cmdline.txt:CMDL;initramfs.cpio.gz:INRD"

### Run Qemu

ranshal@D-P-RANSHAL-L-RF-L-RF:~/projects/SBL/slimbootloader$

ranshal@D-P-RANSHAL-L-RF-L-RF:~/projects/SBL/slimbootloader$ qemu-system-x86_64 -m 512M -machine q35 -serial mon:stdio -nographic   -pflash Outputs/qemu/SlimBootloader.bin

# Linux SBL → OSLoader → OS container (Linux)

### Build SBL

git diff

diff --git a/Platform/QemuBoardPkg/BoardConfig.py b/Platform/QemuBoardPkg/BoardConfig.py

index 61f6e156..a76bba2e 100644

--- a/Platform/QemuBoardPkg/BoardConfig.py

+++ b/Platform/QemuBoardPkg/BoardConfig.py

@@ -118,7 +118,7 @@ class Board(BaseBoard):

self.SBLRSVD_SIZE         = 0x00001000

self.FWUPDATE_SIZE        = 0x00019000 if self.ENABLE_FWU else 0

self.SETUP_SIZE           = 0x00020000 if self.ENABLE_SBL_SETUP else 0

-

+        self.PLD_HEAP_SIZE        = 0x09000000

self._REDUNDANT_LAYOUT    = 1

if not self._REDUNDANT_LAYOUT:

self.TOP_SWAP_SIZE      = 0x000000

ranshal@D-P-RANSHAL-L-RF-L-RF:~/projects/SBL/slimbootloader$

python3 BuildLoader.py build qemu -p "OsLoader.efi:LLDR:Lz4"

### Prepare OS container

echo "root=/dev/sda2 rw console=ttyS0,115200 earlyprintk=serial debug loglevel=7" > cmdline.txt

ranshal@D-P-RANSHAL-L-RF-L-RF:~/projects/SBL/linux$ cat cmdline.txt

root=/dev/sda2 rw console=ttyS0,115200 earlyprintk=serial debug loglevel=7

python3 ../slimbootloader/BootloaderCorePkg/Tools/GenContainer.py create -cl CMDL:cmdline.txt KRNL:./vmlinuz-5.4.0-84-generic INRD:./initrd.img-5.4.0-84-generic -k ../SblKeys/OS1_TestKey_Priv_RSA3072.pem -a RSA3072_PKCS1_SHA2_384 -t CLASSIC -o container

# create a 64 MB FAT image

dd if=/dev/zero of=fat.img bs=1M count=64

mkfs.vfat fat.img

# mount it

sudo mount -o loop fat.img mnt

sudo cp sbl_os mnt

sudo mv mnt/sbl_os mnt/container.bin

sudo umount mnt

### Run Qemu

ranshal@D-P-RANSHAL-L-RF-L-RF:~/projects/SBL/slimbootloader$

ranshal@D-P-RANSHAL-L-RF-L-RF:~/projects/SBL/slimbootloader$ qemu-system-x86_64 -m 512M -machine q35 -serial mon:stdio -nographic   -pflash Outputs/qemu/SlimBootloader.bin   -drive file=../linux/fat.img,format=raw,if=ide

WARNING: Image format was not specified for 'Outputs/qemu/SlimBootloader.bin' and probing guessed raw.

Automatically detecting the format is dangerous for raw images, write operations on block 0 will be restricted.

Specify the 'raw' format explicitly to remove the restrictions.

============= Intel Slim Bootloader STAGE1A =============

SBID: SB_QEMU

ISVN: 001

IVER: 001.000.001.001.37984

SVER: EBC8F36D84A03A4E-dirty

FDBG: BLD(D IA32) FSP(R)

FSPV: ID($QEMFSP$) REV(00001000)

CPUV: ID(60FB1) UCODE(0)

Loader global data @ 0x00001CC0

Run  STAGE1A @ 0x00070000

Load STAGE1B @ 0x0003E000

HASH verification for usage (0x00000001) with Hash Alg (0x2): Success

============= Intel Slim Bootloader STAGE1B =============

Host Bridge Device ID:0x29C0

Board ID:0x1 - Loading QEMU!

QEMU Flash: Attempting flash detection at FFC00000

QemuFlashDetected => FD behaves as FLASH

QemuFlashDetected => Yes

SpiInstance = 0000E460

Variable region: 0xFFC9F000:0x2000

Registering container KEYH

HASH verification for usage (0x00000100) with Hash Alg (0x2): Success

SignType (0x2) SignSize (0x180)  SignHashAlg (0x2)

RSA verification for usage (0x00000100): Success

Verify Container KEYH AuthTypes

Component _HS_ AuthType 2

AppendHashStore KeyHash Component Base = 0xFFFDF370

AppendHashStore KeyHash Component Data Base = 0xFFFDF380

Loading Component KEYH:_HS_

HASH verification for usage (0x00000000) with Hash Alg (0x2): Success

Append public key hash into store: Success

Load EXT CFG Data @ 0x0000EB4C:0x015C ... Success

HASH verification for usage (0x00000200) with Hash Alg (0x2): Success

SignType (0x2) SignSize (0x180)  SignHashAlg (0x2)

RSA verification for usage (0x00000200): Success

BOOT: BP0

MODE: 0

BoardID: 0x01

PlatformName: QEMU_01

Memory Init

Found Config TAG: 0x180 @ 0x0000ECC8

MemCfg.Test1=11223344, MemCfg.Test2=11223346

Found Config TAG: 0x200 @ 0x0000ECD8

SilCfg.Test1=11223347, SilCfg.Test2=04030201

Call FspMemoryInit ... Success

FspMultiPhaseMemInit() returned EFI_UNSUPPORTED. This is expected for FSP 2.3 and older.

Loader global data @ 0x1EEBFD00

Load page table from memory @ 0x1EEBD000

Remapping Stage to 0x1EE8B000

FSP Resource HOB Range        Type       Owner

================================= ==== ====================================

0000000000000000-00000000000A0000  00  00000000-0000-0000-0000-000000000000

00000000000A0000-0000000000100000  05  00000000-0000-0000-0000-000000000000

0000000000100000-000000001EF00000  00  00000000-0000-0000-0000-000000000000

000000001EF00000-000000001F000000  05  69A79759-1373-4367-A6C4-C7F59EFD986E

000000001F000000-0000000020000000  05  D038747C-D00C-4980-B319-490199A47D55

Memory Tolum @ 0x1EF00000

Memory Touum @ 0x100000000

Memory Tom   @ 0x1EF00000

Switch to memory stack @ 0x1EEFFF00

Stage1 stack: 0x2000 (0x1134 used)

Stage1 heap: 0xE000 (0x1FA0 used, 0x31F4 max used)

Call FspTempRamExit ... Success

Memory FSP @ 0x1EF00000

Memory TOP @ 0x1E700000

Loading Component FLMP:SG02

HASH verification for usage (0x00000002) with Hash Alg (0x2): Success

Loaded STAGE2 @ 0x1EE21000

============= Intel Slim Bootloader STAGE2 =============

Unmapping Stage

Board GPIO Init

Get base platform GPIO table from board ID 0

Programming 7 GPIO entries

GPIO GPP_A00 DATA: 0x00000000 0x00000010

GPIO GPP_A02 DATA: 0x80000201 0x00000012

GPIO GPP_A03 DATA: 0xC0000301 0x00000013

GPIO GPP_A04 DATA: 0x00000400 0x00000014

GPIO GPP_A05 DATA: 0x40000500 0x00000015

GPIO GPP_A06 DATA: 0x80000601 0x00000016

GPIO GPP_A07 DATA: 0xC0000700 0x00000017

Test variable services

Loading Component IPFW:TST3

Registering container IPFW

HASH verification for usage (0x00001000) with Hash Alg (0x2): Success

SignType (0x2) SignSize (0x180)  SignHashAlg (0x2)

RSA verification for usage (0x00001000): Success

Verify Container IPFW AuthTypes

Component TST1 AuthType 0

All container content must be authenticated in verified boot flow.

Registering container failed due to Security Violation

Load IP firmware @ 0:0x0 - Unsupported

Silicon Init

Select VBT ImageId 0x00000001

Call FspSiliconInit ...

Success

FspMultiPhaseSiliconInitHandler() returned EFI_UNSUPPORTED.

Graphics Info: 800 x 600 x 32 @ 0x80000000

MEM: 0000000000000000 00000000000A0000 00 1

MEM: 00000000000A0000 0000000000060000 00 2

MEM: 0000000000100000 000000001E58C000 00 1

MEM: 000000001E68C000 0000000000004000 01 2

MEM: 000000001E690000 0000000000068000 00 3

MEM: 000000001E6F8000 0000000000008000 00 4

MEM: 000000001E700000 0000000000800000 00 2

MEM: 000000001EF00000 0000000000100000 00 2

MEM: 000000001F000000 0000000001000000 00 2

MEM: 00000000FFC00000 0000000000400000 00 2

MP Init (Wakeup)

MP Init (Run)

Detected 1 CPU threads

CPU  0 APIC ID: 0

SMM rebase done on 1 CPUs

PCI Enum

Update device table: type (0x9) instance (0) from 0x0 to 0x100

GFX framebuffer WC is enabled @ 0x81000000:0x1000000

Call FspNotifyPhase(20) ... Success

ACPI Init

Publish ACPI table: FACP @ 0x1E690210

Publish ACPI table: HPET @ 0x1E690360

Publish ACPI table: APIC @ 0x1E6903A0

Publish ACPI table: MCFG @ 0x1E690400

Publish ACPI table: FPDT @ 0x1E690440

Publish ACPI table: BGRT @ 0x1E692100

Publish ACPI table: TEST @ 0x1E692140

ACPI Ret: Success

Enable SMRR

Loading Payload ID PYLD

Loading Component FLMP:PYLD

HASH verification for usage (0x00000004) with Hash Alg (0x2): Success

Load Payload ID 0x00000000 @ 0x1EC77000

PE32 Format Payload

HOB @ 0x1EEC0000

Created 4 OS boot options (Current: 31)

TPM Lib Private Data not found

Unable to get log area for TCG 2.0 format events !!

Stage2 stack: 0x40000 (stack used 0xCC4, HOB used 0x15C8, 0x3DD74 free)

Stage2 heap: 0x7C0000 (0x2494D0 used, 0x576B30 free, 0x279613 max used)

Updated SBL Performance Table: S1 = 314000000ns, S2 = 347000000ns, OSL = 8000000ns

Payload entry: 0x1ECA6670

Jump to payload

Payload startup

ACPI PmTimer Base: 0x408

PCI Express  Base: 0xE0000000

====================Os Loader====================

Press any key within 1 second(s) to enter the command shell

Boot options (in HEX):

Idx|ImgType|DevType|DevNum|Flags|HwPart|FsType|SwPart|File/Lbaoffset

0|      0|    SD |    0 |   0 |    0 | AUTO |    0 | container.bin *Current

1|      0|  SATA |    0 |   0 |   FF | AUTO |    0 | container.bin

2|      0|  NVME |    0 |   0 |    0 | AUTO |    0 | container.bin

3|      0|   USB |    0 |   0 |    0 | AUTO |    0 | container.bin

======== Try Booting with Boot Option 0 ========

BootMediumPciBase(0x300)

Getting boot image from SD

MMC global data init

Use SDMA instead of ADMA2

MMC Phase 1 init

SdMmcHcReset: reset done with Time out

SdMmcHcReset Fail Status = 0x80000012

Failed to init media - Time out

Failed to Initialize Boot Device - Type 1, Instance 0

Payload normal heap: 0x9000000 (0xB000 used)

Payload reserved heap: 0x4000 (0x0 used)

Payload stack: 0x10000 (0x4F0 used)

======== Try Booting with Boot Option 1 ========

BootMediumPciBase(0x1F02)

Getting boot image from SATA

Init AHCI controller E00FA000

AHCI port [0] has a [harddisk]

AHCI port [2] has a [cdrom]

Try HwPart 255

Try to find boot partition for HW part 255

Partition type: UNKNOWN  (1 logical partitions)

Find partition success

BootSlot = 0x0

Detected FAT on HwDev 255 Part 0

File 'container.bin' size 52745776

Get file 'container.bin' (size:0x324D630) success.

CloseFileSystem: FsType 0

LoadBootImage ImageType-0 Success

ParseBootImage ImageType-0

Registering container BOOT

HASH verification for usage (0x00000800) with Hash Alg (0x2): Success

SignType (0x1) SignSize (0x180)  SignHashAlg (0x2)

RSA verification for usage (0x00000800): Success

HASH verification for usage (0x00000000) with Hash Alg (0x2): Success

CONTAINER size = 0x324D630, image type = 0xF3, # of components = 4

COMP:CMDL Success

COMP:KRNL Success

COMP:INRD Success

Unregister done - Success!

SetupBootImage ImageType-0

Assume BzImage...

Found bzimage Signature

Dump normal boot image info:

============ KERNEL SETUP ============

SetupSectorss: 0x22

RootFlags: 0x1

SysSize: 0x90038

RamSize: 0x0

VideoMode: 0xFFFF

RootDev: 0x0

Signature: 0xAA55

Jump: 0x66EB

Header: 0x53726448

Version: 0x20D

SuSwitch: 0x0

SetupSeg: 0x0

StartSys: 0x1000

KernalVer: 0x37C0

LoaderId: 0xFF

LoadFlags: 0x1

MoveSize: 0x8000

Code32Start: 0x100000

BootsectorKludge: 0x0

HeapEnd: 0x5A50

ExtLoaderVer: 0x0

ExtLoaderType: 0x0

CmdlineSize: 0x800

CmdLinePtr: 0x1E4EC000

Cmd Args: root=/dev/sda2 rw console=ttyS0,115200 earlyprintk=serial debug loglevel=7

RamDiskStart: 0x1BB94000

RamDisklen: 0x2947611

RamDiskMax: 0x7FFFFFFF

KernelAlignment: 0x200000

RelocatableKernel: 0x1

MinAlignment: 0x15

XloadFlags: 0x3F

HardwareSubarch: 0x0

HardwareSubarchData: 0x0

PayloadOffset: 0x3B1

PayloadLength: 0x8C011D

SetupData: 0x0

PrefAddress: 0x1000000

InitSize: 0x2B6E000

HandoverOffset: 0x190

Payload normal heap: 0x9000000 (0x3362000 used)

Payload reserved heap: 0x4000 (0x0 used)

Payload stack: 0x10000 (0xCB8 used)

MP Init (Done)

Call FspNotifyPhase(40) ... Success

Call FspNotifyPhase(F0) ... Success

Updated SBL Performance Table: S1 = 314000000ns, S2 = 347000000ns, OSL = 61161000000ns

Deinit AHCI controller 0

Not a AHCI controller or Disabled

Id   | Time (ms)  | Delta (ms) | Description

------+------------+------------+----------------------------------

1000 |       1 ms |       1 ms | Reset vector

1010 |      16 ms |      15 ms | Stage1A entry point

1040 |      20 ms |       4 ms | Board PostTempRamInit hook

1060 |      26 ms |       6 ms | Stage1A continuation

1080 |      33 ms |       7 ms | Load Stage1B

10A0 |      88 ms |      55 ms | Verify Stage1B

10B0 |      90 ms |       2 ms | Decompress Stage1B

2000 |      90 ms |       0 ms | Stage1B entry point

2020 |     168 ms |      78 ms | Board PreMemoryInit hook

2030 |     202 ms |      34 ms | FSP MemoryInit

2040 |     212 ms |      10 ms | Board PostMemoryInit hook

2050 |     214 ms |       2 ms | Board PreTempRamExit hook

2060 |     218 ms |       4 ms | FSP TempRamExit

2070 |     218 ms |       0 ms | Board PostTempRamExit hook

2080 |     219 ms |       1 ms | Load Stage2

2090 |     221 ms |       2 ms | Copy Stage2 to memory

20A0 |     307 ms |      86 ms | Verify Stage2

20B0 |     311 ms |       4 ms | Decompress Stage2

20C0 |     311 ms |       0 ms | Extend Stage2 hash

20D0 |     312 ms |       1 ms | Rebase Stage2

3000 |     315 ms |       3 ms | Stage2 entry point

3010 |     362 ms |      47 ms | Board PreSiliconInit hook

3020 |     363 ms |       1 ms | Save NVS data

3030 |     390 ms |      27 ms | FSP SiliconInit

3040 |     392 ms |       2 ms | Board PostSiliconInit hook

3050 |     404 ms |      12 ms | Display splash

3060 |     410 ms |       6 ms | MP wake up

3080 |     412 ms |       2 ms | MP init run

3090 |     415 ms |       3 ms | Board PrePciEnumeration hook

30A0 |     428 ms |      13 ms | PCI enumeration

30B0 |     434 ms |       6 ms | Board PostPciEnumeration hook

30C0 |     435 ms |       1 ms | FSP PostPciEnumeration notify

30D0 |     443 ms |       8 ms | ACPI init

30E0 |     444 ms |       1 ms | Board PrePayloadLoading hook

3100 |     445 ms |       1 ms | Load payload

3110 |     445 ms |       0 ms | Locate payload

3120 |     446 ms |       1 ms | Copy payload to memory

3130 |     561 ms |     115 ms | Verify payload

3140 |     659 ms |      98 ms | Decompress payload

3150 |     659 ms |       0 ms | Extend payload hash

31A0 |     659 ms |       0 ms | Board PostPayloadLoading hook

31B0 |     662 ms |       3 ms | Decode payload format

31F0 |     662 ms |       0 ms | End of stage2

4000 |     671 ms |       9 ms | Payload entry point

4010 |     685 ms |      14 ms | OS loader main entry

4020 |    7986 ms |    7301 ms | Shell exit

4040 |    7989 ms |       3 ms | Load image entry

4040 |   12991 ms |    5002 ms | Load image entry

4050 |   13222 ms |     231 ms | Boot device init

4055 |   13222 ms |       0 ms | Boot device tuning

4060 |   13232 ms |      10 ms | Parse partition info

4070 |   13622 ms |     390 ms | Load boot images

4080 |   13657 ms |      35 ms | Verify boot image

4080 |   61775 ms |   48118 ms | Verify boot image

40A0 |   61778 ms |       3 ms | Process boot image type

40B0 |   61814 ms |      36 ms | Process ELF/MultiBoot

40E0 |   61820 ms |       6 ms | Kernel setup

4100 |   61823 ms |       3 ms | FSP ReadyToBoot/EndOfFirmware notify

------+------------+------------+----------------------------------

Starting Kernel ...

Switch to LongMode and jump to 64-bit kernel entrypoint ...

[    0.000000] Linux version 5.4.0-84-generic (buildd@lcy01-amd64-007) (gcc version 7.5.0 (Ubuntu 7.5.0-3ubuntu1~18.04)) #94~18.04.1-Ubuntu SMP Thu Aug 26 23:17:46 UTC 2021 (Ubuntu 5.4.0-84.94~18.04.1-generic 5.4.133)

[    0.000000] Command line: root=/dev/sda2 rw console=ttyS0,115200 earlyprintk=serial debug loglevel=7 acpi_rsdp=0x1E690000

[    0.000000] KERNEL supported cpus:

[    0.000000]   Intel GenuineIntel

[    0.000000]   AMD AuthenticAMD

[    0.000000]   Hygon HygonGenuine

[    0.000000]   Centaur CentaurHauls

[    0.000000]   zhaoxin   Shanghai

[    0.000000] x86/fpu: x87 FPU will use FXSAVE

[    0.000000] BIOS-provided physical RAM map:

[    0.000000] BIOS-e820: [mem 0x0000000000000000-0x000000000009ffff] usable

[    0.000000] BIOS-e820: [mem 0x00000000000a0000-0x00000000000fffff] reserved

[    0.000000] BIOS-e820: [mem 0x0000000000100000-0x000000001e68bfff] usable

[    0.000000] BIOS-e820: [mem 0x000000001e68c000-0x000000001e68ffff] reserved

[    0.000000] BIOS-e820: [mem 0x000000001e690000-0x000000001e6f7fff] ACPI data

[    0.000000] BIOS-e820: [mem 0x000000001e6f8000-0x000000001e6fffff] ACPI NVS

[    0.000000] BIOS-e820: [mem 0x000000001e700000-0x000000001fffffff] reserved

[    0.000000] BIOS-e820: [mem 0x00000000ffc00000-0x00000000ffffffff] reserved

[    0.000000] printk: bootconsole [earlyser0] enabled

[    0.000000] NX (Execute Disable) protection: active

[    0.000000] SMBIOS 3.3 present.

[    0.000000] DMI: Intel Corporation QEMU Virtual Platform/ QEMU_01 (ID:01), BIOS SB_QEMU.001.000.001.001.37984.D Sep  1 2025

[    0.000000] tsc: Fast TSC calibration using PIT

[    0.000000] tsc: Detected 2918.387 MHz processor

[    0.002366] AGP: No AGP bridge found

[    0.002602] last_pfn = 0x1e68c max_arch_pfn = 0x400000000

[    0.003814] x86/PAT: Configuration [0-7]: WB  WC  UC- UC  WB  WP  UC- WT

Memory KASLR using RDTSC...

[    0.022598] check: Scanning 1 areas for low memory corruption

[    0.025869] RAMDISK: [mem 0x1bb94000-0x1e4dbfff]

[    0.026306] ACPI: Early table checksum verification disabled

[    0.026792] ACPI: RSDP 0x000000001E690000 000024 (v02 OEMID )

[    0.027058] ACPI: XSDT 0x000000001E6900E0 00005C (v01 OEMID  OEMTABLE 00000005 CREA 0100000D)

[    0.027692] ACPI: FACP 0x000000001E690210 00010C (v05 OEMID  OEMTABLE 00000005 CREA 0100000D)

[    0.028300] ACPI: DSDT 0x000000001E690520 001BD9 (v02 OEMID  APL-SOC  00000000 INTL 20200925)

[    0.028457] ACPI: FACS 0x000000001E690320 000040

[    0.028538] ACPI: FACS 0x000000001E690320 000040

[    0.028633] ACPI: HPET 0x000000001E690360 000038 (v01 OEMID  OEMTABLE 00000005 CREA 0100000D)

[    0.028751] ACPI: APIC 0x000000001E6903A0 00005A (v03                 00000000      00000000)

[    0.028865] ACPI: MCFG 0x000000001E690400 00003C (v01                 00000001      00000000)

[    0.028992] ACPI: FPDT 0x000000001E690440 0000D4 (v01 INTEL  OEMTABLE 00000005 CREA 0100000D)

[    0.029079] ACPI: BGRT 0x000000001E692100 000038 (v01 OEMID  OEMTABLE 00000005 CREA 0100000D)

[    0.029202] ACPI: TEST 0x000000001E692140 00002C (v01 OEMID  OEMTABLE 00000001 CREA 01000001)

[    0.029392] ACPI: Reserving FACP table memory at [mem 0x1e690210-0x1e69031b]

[    0.029508] ACPI: Reserving DSDT table memory at [mem 0x1e690520-0x1e6920f8]

[    0.029608] ACPI: Reserving FACS table memory at [mem 0x1e690320-0x1e69035f]

[    0.029705] ACPI: Reserving FACS table memory at [mem 0x1e690320-0x1e69035f]

[    0.029797] ACPI: Reserving HPET table memory at [mem 0x1e690360-0x1e690397]

[    0.029880] ACPI: Reserving APIC table memory at [mem 0x1e6903a0-0x1e6903f9]

[    0.029964] ACPI: Reserving MCFG table memory at [mem 0x1e690400-0x1e69043b]

[    0.030054] ACPI: Reserving FPDT table memory at [mem 0x1e690440-0x1e690513]

[    0.030147] ACPI: Reserving BGRT table memory at [mem 0x1e692100-0x1e692137]

[    0.030207] ACPI: Reserving TEST table memory at [mem 0x1e692140-0x1e69216b]

[    0.032113] No NUMA configuration found

[    0.032172] Faking a node at [mem 0x0000000000000000-0x000000001e68bfff]

[    0.032856] NODE_DATA(0) allocated [mem 0x1e661000-0x1e68bfff]

[    0.037046] Zone ranges:

[    0.037103]   DMA      [mem 0x0000000000001000-0x0000000000ffffff]

[    0.037218]   DMA32    [mem 0x0000000001000000-0x000000001e68bfff]

[    0.037306]   Normal   empty

[    0.037356]   Device   empty

[    0.037403] Movable zone start for each node

[    0.037489] Early memory node ranges

[    0.037558]   node   0: [mem 0x0000000000001000-0x000000000009ffff]

[    0.037762]   node   0: [mem 0x0000000000100000-0x000000001e68bfff]

[    0.038251] Zeroed struct page in unavailable ranges: 6613 pages

[    0.038386] Initmem setup node 0 [mem 0x0000000000001000-0x000000001e68bfff]

[    0.045853] ACPI: PM-Timer IO Port: 0x408

[    0.046301] ACPI: LAPIC_NMI (acpi_id[0xff] high level lint[0x1])

[    0.046719] IOAPIC[0]: apic_id 1, version 32, address 0xfec00000, GSI 0-23

[    0.046908] ACPI: INT_SRC_OVR (bus 0 bus_irq 0 global_irq 2 dfl dfl)

[    0.047210] ACPI: INT_SRC_OVR (bus 0 bus_irq 9 global_irq 9 low level)

[    0.047534] Using ACPI (MADT) for SMP configuration information

[    0.047655] ACPI: HPET id: 0x0 base: 0xfed00000

[    0.048034] smpboot: Allowing 1 CPUs, 0 hotplug CPUs

[    0.048781] PM: Registered nosave memory: [mem 0x00000000-0x00000fff]

[    0.048908] PM: Registered nosave memory: [mem 0x000a0000-0x000fffff]

[    0.049088] [mem 0x20000000-0xffbfffff] available for PCI devices

[    0.049200] Booting paravirtualized kernel on bare hardware

[    0.049582] clocksource: refined-jiffies: mask: 0xffffffff max_cycles: 0xffffffff, max_idle_ns: 7645519600211568 ns

[    0.050031] setup_percpu: NR_CPUS:8192 nr_cpumask_bits:1 nr_cpu_ids:1 nr_node_ids:1

[    0.053913] percpu: Embedded 55 pages/cpu s188416 r8192 d28672 u2097152

[    0.055630] Built 1 zonelists, mobility grouping on.  Total pages: 122421

[    0.055736] Policy zone: DMA32

[    0.055940] Kernel command line: root=/dev/sda2 rw console=ttyS0,115200 earlyprintk=serial debug loglevel=7 acpi_rsdp=0x1E690000

[    0.057661] Dentry cache hash table entries: 65536 (order: 7, 524288 bytes, linear)

[    0.058338] Inode-cache hash table entries: 32768 (order: 6, 262144 bytes, linear)

[    0.059720] mem auto-init: stack:off, heap alloc:on, heap free:off

[    0.060070] AGP: Checking aperture...

[    0.060818] AGP: No AGP bridge found

[    0.064814] Memory: 412976K/497836K available (14339K kernel code, 2381K rwdata, 4984K rodata, 2716K init, 5004K bss, 84860K reserved, 0K cma-reserved)

[    0.065516] random: get_random_u64 called from __kmem_cache_create+0x41/0x550 with crng_init=0

[    0.071256] SLUB: HWalign=64, Order=0-3, MinObjects=0, CPUs=1, Nodes=1

[    0.072655] ftrace: allocating 44320 entries in 174 pages

[    0.136261] rcu: Hierarchical RCU implementation.

[    0.136408] rcu:     RCU restricting CPUs from NR_CPUS=8192 to nr_cpu_ids=1.

[    0.136547]     Tasks RCU enabled.

[    0.136630] rcu: RCU calculated value of scheduler-enlistment delay is 25 jiffies.

[    0.136745] rcu: Adjusting geometry for rcu_fanout_leaf=16, nr_cpu_ids=1

[    0.149268] NR_IRQS: 524544, nr_irqs: 256, preallocated irqs: 16

[    0.157730] Console: colour dummy device 80x25

[    0.158961] printk: console [ttyS0] enabled

[    0.158961] printk: console [ttyS0] enabled

[    0.159293] printk: bootconsole [earlyser0] disabled

[    0.159293] printk: bootconsole [earlyser0] disabled

[    0.160898] ACPI: Core revision 20190816

[    0.165411] clocksource: hpet: mask: 0xffffffff max_cycles: 0xffffffff, max_idle_ns: 19112604467 ns

[    0.166865] APIC: Switch to symmetric I/O mode setup

[    0.170642] ..TIMER: vector=0x30 apic1=0 pin1=2 apic2=-1 pin2=-1

[    0.190578] clocksource: tsc-early: mask: 0xffffffffffffffff max_cycles: 0x2a111c84998, max_idle_ns: 440795323206 ns

[    0.191238] Calibrating delay loop (skipped), value calculated using timer frequency.. 5836.77 BogoMIPS (lpj=11673548)

[    0.191454] pid_max: default: 32768 minimum: 301

[    0.192156] LSM: Security Framework initializing

[    0.192916] Yama: becoming mindful.

[    0.195375] AppArmor: AppArmor initialized

[    0.196720] Mount-cache hash table entries: 1024 (order: 1, 8192 bytes, linear)

[    0.196815] Mountpoint-cache hash table entries: 1024 (order: 1, 8192 bytes, linear)

[    0.198047] *** VALIDATE tmpfs ***

[    0.204731] *** VALIDATE proc ***

[    0.207963] *** VALIDATE cgroup1 ***

[    0.208022] *** VALIDATE cgroup2 ***

Poking KASLR using RDTSC...

[    0.211553] process: using AMD E400 aware idle routine

[    0.211757] Last level iTLB entries: 4KB 512, 2MB 255, 4MB 127

[    0.211821] Last level dTLB entries: 4KB 512, 2MB 255, 4MB 127, 1GB 0

[    0.212086] Spectre V1 : Mitigation: usercopy/swapgs barriers and __user pointer sanitization

[    0.212252] Spectre V2 : Mitigation: Full AMD retpoline

[    0.212310] Spectre V2 : Spectre v2 / SpectreRSB mitigation: Filling RSB on context switch

[    0.376474] Freeing SMP alternatives memory: 40K

[    0.507341] smpboot: CPU0: AMD QEMU Virtual CPU version 2.5+ (family: 0xf, model: 0x6b, stepping: 0x1)

[    0.511046] Performance Events: PMU not available due to virtualization, using software events only.

[    0.519441] rcu: Hierarchical SRCU implementation.

[    0.523592] NMI watchdog: Perf NMI watchdog permanently disabled

[    0.526725] smp: Bringing up secondary CPUs ...

[    0.526822] smp: Brought up 1 node, 1 CPU

[    0.526899] smpboot: Max logical packages: 1

[    0.527405] smpboot: Total of 1 processors activated (5836.77 BogoMIPS)

[    0.536142] devtmpfs: initialized

[    0.538428] x86/mm: Memory block size: 128MB

[    0.541639] PM: Registering ACPI NVS region [mem 0x1e6f8000-0x1e6fffff] (32768 bytes)

[    0.542669] clocksource: jiffies: mask: 0xffffffff max_cycles: 0xffffffff, max_idle_ns: 7645041785100000 ns

[    0.542893] futex hash table entries: 256 (order: 2, 16384 bytes, linear)

[    0.544855] pinctrl core: initialized pinctrl subsystem

[    0.549830] PM: RTC time: 13:26:29, date: 2025-09-01

[    0.554741] NET: Registered protocol family 16

[    0.556484] audit: initializing netlink subsys (disabled)

[    0.559513] EISA bus registered

[    0.559665] cpuidle: using governor ladder

[    0.559814] cpuidle: using governor menu

[    0.560684] ACPI: bus type PCI registered

[    0.560793] acpiphp: ACPI Hot Plug PCI Controller Driver version: 0.5

[    0.562510] PCI: MMCONFIG for domain 0000 [bus 00-ff] at [mem 0xe0000000-0xefffffff] (base 0xe0000000)

[    0.562761] PCI: not using MMCONFIG

[    0.562871] PCI: Using configuration type 1 for base access

[    0.563376] audit: type=2000 audit(1756733189.384:1): state=initialized audit_enabled=0 res=1

[    0.576243] HugeTLB registered 2.00 MiB page size, pre-allocated 0 pages

[    0.588982] ACPI: Added _OSI(Module Device)

[    0.589051] ACPI: Added _OSI(Processor Device)

[    0.589094] ACPI: Added _OSI(3.0 _SCP Extensions)

[    0.589152] ACPI: Added _OSI(Processor Aggregator Device)

[    0.589285] ACPI: Added _OSI(Linux-Dell-Video)

[    0.589328] ACPI: Added _OSI(Linux-Lenovo-NV-HDMI-Audio)

[    0.589402] ACPI: Added _OSI(Linux-HPI-Hybrid-Graphics)

[    0.607419] ACPI: 1 ACPI AML tables successfully acquired and loaded

[    0.615716] ACPI: Interpreter enabled

[    0.616320] ACPI: (supports S0 S5)

[    0.616394] ACPI: Using IOAPIC for interrupt routing

[    0.618665] PCI: MMCONFIG for domain 0000 [bus 00-ff] at [mem 0xe0000000-0xefffffff] (base 0xe0000000)

[    0.620729] PCI: MMCONFIG at [mem 0xe0000000-0xefffffff] reserved in ACPI motherboard resources

[    0.621218] PCI: Using host bridge windows from ACPI; if necessary, use "pci=nocrs" and report a bug

[    0.642073] ACPI: PCI Root Bridge [PCI0] (domain 0000 [bus 00-ff])

[    0.642468] acpi PNP0A03:00: _OSC: OS supports [ExtendedConfig ASPM ClockPM Segments MSI HPX-Type3]

[    0.646155] PCI host bridge to bus 0000:00

[    0.646301] pci_bus 0000:00: root bus resource [io  0x0000-0x0cf7 window]

[    0.646387] pci_bus 0000:00: root bus resource [io  0x0d00-0xffff window]

[    0.646449] pci_bus 0000:00: root bus resource [mem 0x000a0000-0x000bffff window]

[    0.646548] pci_bus 0000:00: root bus resource [mem 0x80000000-0xdfffffff window]

[    0.646644] pci_bus 0000:00: root bus resource [mem 0x800000000-0xfffffffff window]

[    0.646825] pci_bus 0000:00: root bus resource [bus 00-ff]

[    0.647473] pci 0000:00:00.0: [8086:29c0] type 00 class 0x060000

[    0.650056] pci 0000:00:01.0: [1234:1111] type 00 class 0x030000

[    0.650842] pci 0000:00:01.0: reg 0x10: [mem 0x81000000-0x81ffffff pref]

[    0.651937] pci 0000:00:01.0: reg 0x18: [mem 0x80045000-0x80045fff]

[    0.654048] pci 0000:00:01.0: reg 0x30: [mem 0x00000000-0x0000ffff pref]

[    0.654283] pci 0000:00:01.0: BAR 0: assigned to efifb

[    0.655283] pci 0000:00:02.0: [8086:10d3] type 00 class 0x020000

[    0.656338] pci 0000:00:02.0: reg 0x10: [mem 0x80020000-0x8003ffff]

[    0.657236] pci 0000:00:02.0: reg 0x14: [mem 0x80000000-0x8001ffff]

[    0.658142] pci 0000:00:02.0: reg 0x18: [io  0x2060-0x207f]

[    0.658968] pci 0000:00:02.0: reg 0x1c: [mem 0x80040000-0x80043fff]

[    0.660812] pci 0000:00:02.0: reg 0x30: [mem 0x00000000-0x0007ffff pref]

[    0.663269] pci 0000:00:1f.0: [8086:2918] type 00 class 0x060100

[    0.663593] pci 0000:00:1f.0: quirk: [io  0x0400-0x047f] claimed by ICH6 ACPI/GPIO/TCO

[    0.663978] pci 0000:00:1f.2: [8086:2922] type 00 class 0x010601

[    0.665551] pci 0000:00:1f.2: reg 0x20: [io  0x2040-0x205f]

[    0.665975] pci 0000:00:1f.2: reg 0x24: [mem 0x80044000-0x80044fff]

[    0.666697] pci 0000:00:1f.3: [8086:2930] type 00 class 0x0c0500

[    0.667645] pci 0000:00:1f.3: reg 0x20: [io  0x2000-0x203f]

[    0.669460] ACPI: PCI Interrupt Link [LNKS] (IRQs *9)

[    0.670379] ACPI: PCI Interrupt Link [LNKA] (IRQs 5 10 11) *0

[    0.671773] ACPI: PCI Interrupt Link [LNKB] (IRQs 5 10 11) *0

[    0.672482] ACPI: PCI Interrupt Link [LNKC] (IRQs 5 10 11) *0

[    0.673155] ACPI: PCI Interrupt Link [LNKD] (IRQs 5 10 11) *0

[    0.676919] iommu: Default domain type: Translated

[    0.678938] SCSI subsystem initialized

[    0.681985] pci 0000:00:01.0: vgaarb: setting as boot VGA device

[    0.682118] pci 0000:00:01.0: vgaarb: VGA device added: decodes=io+mem,owns=io+mem,locks=none

[    0.682236] pci 0000:00:01.0: vgaarb: bridge control possible

[    0.682377] vgaarb: loaded

[    0.682902] ACPI: bus type USB registered

[    0.683125] usbcore: registered new interface driver usbfs

[    0.683355] usbcore: registered new interface driver hub

[    0.683490] usbcore: registered new device driver usb

[    0.683866] pps_core: LinuxPPS API ver. 1 registered

[    0.683922] pps_core: Software ver. 5.3.6 - Copyright 2005-2007 Rodolfo Giometti <giometti@linux.it>

[    0.684042] PTP clock support registered

[    0.684503] EDAC MC: Ver: 3.0.0

[    0.686845] PCI: Using ACPI for IRQ routing

[    0.696219] NetLabel: Initializing

[    0.696274] NetLabel:  domain hash size = 128

[    0.696331] NetLabel:  protocols = UNLABELED CIPSOv4 CALIPSO

[    0.697217] NetLabel:  unlabeled traffic allowed by default

[    0.698727] hpet: 3 channels of 0 reserved for per-cpu timers

[    0.699095] hpet0: at MMIO 0xfed00000, IRQs 2, 8, 0

[    0.699203] hpet0: 3 comparators, 64-bit 100.000000 MHz counter

[    0.702852] clocksource: Switched to clocksource tsc-early

[    0.763863] *** VALIDATE bpf ***

[    0.764539] VFS: Disk quotas dquot_6.6.0

[    0.764727] VFS: Dquot-cache hash table entries: 512 (order 0, 4096 bytes)

[    0.765742] *** VALIDATE ramfs ***

[    0.765833] *** VALIDATE hugetlbfs ***

[    0.767873] AppArmor: AppArmor Filesystem Enabled

[    0.768386] pnp: PnP ACPI init

[    0.770082] system 00:00: [mem 0xe0000000-0xefffffff] has been reserved

[    0.770175] system 00:00: [mem 0xfed1c000-0xfed1cfff] has been reserved

[    0.770246] system 00:00: [mem 0xfee00000-0xfeefffff] has been reserved

[    0.772178] system 00:02: [io  0x01e0-0x01ef] has been reserved

[    0.772256] system 00:02: [io  0x0160-0x016f] has been reserved

[    0.772314] system 00:02: [io  0x0278-0x027f] has been reserved

[    0.772384] system 00:02: [io  0x0370-0x0371] has been reserved

[    0.772460] system 00:02: [io  0x0378-0x037f] has been reserved

[    0.772513] system 00:02: [io  0x0678-0x067f] has been reserved

[    0.772582] system 00:02: [io  0x0778-0x077f] has been reserved

[    0.772672] system 00:02: [io  0x0800] has been reserved

[    0.772721] system 00:02: [io  0xafe0-0xafe3] has been reserved

[    0.772801] system 00:02: [io  0xb000-0xb03f] has been reserved

[    0.773858] pnp: PnP ACPI: found 9 devices

[    0.782790] thermal_sys: Registered thermal governor 'fair_share'

[    0.782812] thermal_sys: Registered thermal governor 'bang_bang'

[    0.782896] thermal_sys: Registered thermal governor 'step_wise'

[    0.782962] thermal_sys: Registered thermal governor 'user_space'

[    0.783054] thermal_sys: Registered thermal governor 'power_allocator'

[    0.787832] clocksource: acpi_pm: mask: 0xffffff max_cycles: 0xffffff, max_idle_ns: 2085701024 ns

[    0.789181] pci 0000:00:02.0: BAR 6: assigned [mem 0x80080000-0x800fffff pref]

[    0.789351] pci 0000:00:01.0: BAR 6: assigned [mem 0x80050000-0x8005ffff pref]

[    0.789599] pci_bus 0000:00: resource 4 [io  0x0000-0x0cf7 window]

[    0.789660] pci_bus 0000:00: resource 5 [io  0x0d00-0xffff window]

[    0.789732] pci_bus 0000:00: resource 6 [mem 0x000a0000-0x000bffff window]

[    0.789824] pci_bus 0000:00: resource 7 [mem 0x80000000-0xdfffffff window]

[    0.789884] pci_bus 0000:00: resource 8 [mem 0x800000000-0xfffffffff window]

[    0.790592] NET: Registered protocol family 2

[    0.791314] IP idents hash table entries: 8192 (order: 4, 65536 bytes, linear)

[    0.794589] tcp_listen_portaddr_hash hash table entries: 256 (order: 0, 4096 bytes, linear)

[    0.794778] TCP established hash table entries: 4096 (order: 3, 32768 bytes, linear)

[    0.795067] TCP bind hash table entries: 4096 (order: 4, 65536 bytes, linear)

[    0.795261] TCP: Hash tables configured (established 4096 bind 4096)

[    0.796261] UDP hash table entries: 256 (order: 1, 8192 bytes, linear)

[    0.796486] UDP-Lite hash table entries: 256 (order: 1, 8192 bytes, linear)

[    0.798799] NET: Registered protocol family 1

[    0.799144] NET: Registered protocol family 44

[    0.799490] pci 0000:00:01.0: Video device with shadowed ROM at [mem 0x000c0000-0x000dffff]

[    0.799699] PCI: CLS 0 bytes, default 64

[    0.802542] Trying to unpack rootfs image as initramfs...

[    1.970313] Freeing initrd memory: 42272K

[    1.971713] check: Scanning for low memory corruption every 60 seconds

[    1.974385] Initialise system trusted keyrings

[    1.975761] Key type blacklist registered

[    1.976260] workingset: timestamp_bits=36 max_order=17 bucket_order=0

[    1.982744] zbud: loaded

[    1.985344] squashfs: version 4.0 (2009/01/31) Phillip Lougher

[    1.986538] fuse: init (API version 7.31)

[    1.986703] *** VALIDATE fuse ***

[    1.986752] *** VALIDATE fuse ***

[    1.988166] Platform Keyring initialized

[    1.993985] Key type asymmetric registered

[    1.994082] Asymmetric key parser 'x509' registered

[    1.994278] Block layer SCSI generic (bsg) driver version 0.4 loaded (major 244)

[    1.994842] io scheduler mq-deadline registered

[    1.996699] shpchp: Standard Hot Plug PCI Controller Driver version: 0.4

[    1.998271] efifb: probing for efifb

[    1.998543] efifb: No BGRT, not showing boot graphics

[    1.998606] efifb: framebuffer at 0x81000000, using 1876k, total 1875k

[    1.998672] efifb: mode is 800x600x32, linelength=3200, pages=1

[    1.998755] efifb: scrolling: redraw

[    1.998824] efifb: Truecolor: size=8:8:8:8, shift=24:16:8:0

[    1.999945] fbcon: Deferring console take-over

[    2.000082] fb0: EFI VGA frame buffer device

[    2.001477] input: Power Button as /devices/LNXSYSTM:00/LNXPWRBN:00/input/input0

[    2.003947] ACPI: Power Button [PWRF]

[    2.005917] Serial: 8250/16550 driver, 32 ports, IRQ sharing enabled

[    2.027994] 00:05: ttyS0 at I/O 0x3f8 (irq = 4, base_baud = 115200) is a 16550A

[    2.038286] Linux agpgart interface v0.103

[    2.143940] loop: module loaded

[    2.150361] libphy: Fixed MDIO Bus: probed

[    2.150555] tun: Universal TUN/TAP device driver, 1.6

[    2.151568] PPP generic driver version 2.4.2

[    2.152778] VFIO - User Level meta-driver version: 0.3

[    2.154287] ehci_hcd: USB 2.0 'Enhanced' Host Controller (EHCI) Driver

[    2.154538] ehci-pci: EHCI PCI platform driver

[    2.154844] ehci-platform: EHCI generic platform driver

[    2.155024] ohci_hcd: USB 1.1 'Open' Host Controller (OHCI) Driver

[    2.155341] ohci-pci: OHCI PCI platform driver

[    2.155654] ohci-platform: OHCI generic platform driver

[    2.155833] uhci_hcd: USB Universal Host Controller Interface driver

[    2.157137] i8042: PNP: PS/2 Controller [PNP0303:PS2K,PNP0f03:PS2M] at 0x60,0x64 irq 1,12

[    2.159723] serio: i8042 KBD port at 0x60,0x64 irq 1

[    2.159926] serio: i8042 AUX port at 0x60,0x64 irq 12

[    2.161088] mousedev: PS/2 mouse device common for all mice

[    2.162965] input: AT Raw Set 2 keyboard as /devices/platform/i8042/serio0/input/input1

[    2.164207] rtc_cmos 00:01: RTC can wake from S4

[    2.166530] rtc_cmos 00:01: registered as rtc0

[    2.167060] rtc_cmos 00:01: alarms up to one month, y3k, 114 bytes nvram, hpet irqs

[    2.167355] i2c /dev entries driver

[    2.167782] device-mapper: uevent: version 1.0.3

[    2.168367] device-mapper: ioctl: 4.41.0-ioctl (2019-09-16) initialised: dm-devel@redhat.com

[    2.168642] platform eisa.0: Probing EISA bus 0

[    2.168795] platform eisa.0: EISA: Cannot allocate resource for mainboard

[    2.168929] platform eisa.0: Cannot allocate resource for EISA slot 1

[    2.169039] platform eisa.0: Cannot allocate resource for EISA slot 2

[    2.169132] platform eisa.0: Cannot allocate resource for EISA slot 3

[    2.169219] platform eisa.0: Cannot allocate resource for EISA slot 4

[    2.169307] platform eisa.0: Cannot allocate resource for EISA slot 5

[    2.169390] platform eisa.0: Cannot allocate resource for EISA slot 6

[    2.169477] platform eisa.0: Cannot allocate resource for EISA slot 7

[    2.169552] platform eisa.0: Cannot allocate resource for EISA slot 8

[    2.169633] platform eisa.0: EISA: Detected 0 cards

[    2.169946] ledtrig-cpu: registered to indicate activity on CPUs

[    2.170442] drop_monitor: Initializing network drop monitor service

[    2.171671] NET: Registered protocol family 10

[    2.311190] Segment Routing with IPv6

[    2.311551] NET: Registered protocol family 17

[    2.312032] Key type dns_resolver registered

[    2.313308] RAS: Correctable Errors collector initialized.

[    2.313651] IPI shorthand broadcast: enabled

[    2.313852] sched_clock: Marking stable (2293498583, 17544881)->(2314823620, -3780156)

[    2.314883] registered taskstats version 1

[    2.315260] Loading compiled-in X.509 certificates

[    2.323171] Loaded X.509 cert 'Build time autogenerated kernel key: 2e5a5cfcbde5ccff8872bb2bdda0161154af6a9e'

[    2.324515] Loaded X.509 cert 'Canonical Ltd. Live Patch Signing: 14df34d1a87cf37625abec039ef2bf521249b969'

[    2.325610] Loaded X.509 cert 'Canonical Ltd. Kernel Module Signing: 88f752e560a1e0737e31163a466ad7b70a850c19'

[    2.326404] zswap: loaded using pool lzo/zbud

[    2.327384] Key type ._fscrypt registered

[    2.327448] Key type .fscrypt registered

[    2.430940] Key type big_key registered

[    2.479445] Key type encrypted registered

[    2.479578] AppArmor: AppArmor sha1 policy hashing enabled

[    2.479922] ima: No TPM chip found, activating TPM-bypass!

[    2.480069] ima: Allocated hash algorithm: sha1

[    2.481303] ima: No architecture policies found

[    2.481603] evm: Initialising EVM extended attributes:

[    2.481686] evm: security.selinux

[    2.481741] evm: security.SMACK64

[    2.481787] evm: security.SMACK64EXEC

[    2.481838] evm: security.SMACK64TRANSMUTE

[    2.481894] evm: security.SMACK64MMAP

[    2.481944] evm: security.apparmor

[    2.481991] evm: security.ima

[    2.482033] evm: security.capability

[    2.482087] evm: HMAC attrs: 0x1

[    2.484081] PM:   Magic number: 9:844:431

[    2.484818] rtc_cmos 00:01: setting system clock to 2025-09-01T13:26:31 UTC (1756733191)

[    2.486784] powernow_k8: Power state transitions not supported

[    2.495130] Freeing unused decrypted memory: 2040K

[    2.541986] Freeing unused kernel image memory: 2716K

[    2.542305] Write protecting the kernel read-only data: 22528k

[    2.543969] Freeing unused kernel image memory: 2008K

[    2.544637] Freeing unused kernel image memory: 1160K

[    2.577888] x86/mm: Checked W+X mappings: passed, no W+X pages found.

[    2.578056] Run /init as init process

Loading, please wait...

[    2.892428] systemd-udevd[129]: starting version 237

[    2.987208] tsc: Refined TSC clocksource calibration: 2918.364 MHz

[    2.987415] clocksource: tsc: mask: 0xffffffffffffffff max_cycles: 0x2a1106ea954, max_idle_ns: 440795315443 ns

[    2.987710] clocksource: Switched to clocksource tsc

[    3.738611] e1000e: Intel(R) PRO/1000 Network Driver - 3.2.6-k

[    3.738703] e1000e: Copyright(c) 1999 - 2015 Intel Corporation.

[    3.788956] e1000e 0000:00:02.0: Interrupt Throttling Rate (ints/sec) set to dynamic conservative mode

[    3.794541] ahci 0000:00:1f.2: AHCI 0001.0000 32 slots 6 ports 1.5 Gbps 0x3f impl SATA mode

[    3.794730] ahci 0000:00:1f.2: flags: 64bit ncq only

[    3.842020] scsi host0: ahci

[    3.855009] scsi host1: ahci

[    3.856386] e1000e 0000:00:02.0 0000:00:02.0 (uninitialized): registered PHC clock

[    3.865936] scsi host2: ahci

[    3.873636] scsi host3: ahci

[    3.881714] scsi host4: ahci

[    3.889683] scsi host5: ahci

[    3.890378] ata1: SATA max UDMA/133 abar m4096@0x80044000 port 0x80044100 irq 27

[    3.890485] ata2: SATA max UDMA/133 abar m4096@0x80044000 port 0x80044180 irq 27

[    3.890617] ata3: SATA max UDMA/133 abar m4096@0x80044000 port 0x80044200 irq 27

[    3.890703] ata4: SATA max UDMA/133 abar m4096@0x80044000 port 0x80044280 irq 27

[    3.890787] ata5: SATA max UDMA/133 abar m4096@0x80044000 port 0x80044300 irq 27

[    3.890882] ata6: SATA max UDMA/133 abar m4096@0x80044000 port 0x80044380 irq 27

[    3.923256] e1000e 0000:00:02.0 eth0: (PCI Express:2.5GT/s:Width x1) 52:54:00:12:34:56

[    3.923376] e1000e 0000:00:02.0 eth0: Intel(R) PRO/1000 Network Connection

[    3.923552] e1000e 0000:00:02.0 eth0: MAC: 3, PHY: 8, PBA No: 000000-000

[    4.122449] e1000e 0000:00:02.0 enp0s2: renamed from eth0

[    4.217879] ata1: SATA link up 1.5 Gbps (SStatus 113 SControl 300)

[    4.220148] ata6: SATA link down (SStatus 0 SControl 300)

[    4.220793] ata5: SATA link down (SStatus 0 SControl 300)

[    4.220885] ata4: SATA link down (SStatus 0 SControl 300)

[    4.220981] ata3: SATA link up 1.5 Gbps (SStatus 113 SControl 300)

[    4.221822] ata3.00: ATAPI: QEMU DVD-ROM, 2.5+, max UDMA/100

[    4.221937] ata3.00: applying bridge limits

[    4.222795] ata3.00: configured for UDMA/100

[    4.223264] ata2: SATA link down (SStatus 0 SControl 300)

[    4.224056] ata1.00: ATA-7: QEMU HARDDISK, 2.5+, max UDMA/100

[    4.224124] ata1.00: 131072 sectors, multi 16: LBA48 NCQ (depth 32)

[    4.224247] ata1.00: applying bridge limits

[    4.224387] ata1.00: configured for UDMA/100

[    4.229884] scsi 0:0:0:0: Direct-Access     ATA      QEMU HARDDISK    2.5+ PQ: 0 ANSI: 5

[    4.234039] scsi 0:0:0:0: Attached scsi generic sg0 type 0

[    4.237327] scsi 2:0:0:0: CD-ROM            QEMU     QEMU DVD-ROM     2.5+ PQ: 0 ANSI: 5

[    4.240946] sd 0:0:0:0: [sda] 131072 512-byte logical blocks: (67.1 MB/64.0 MiB)

[    4.241529] sd 0:0:0:0: [sda] Write Protect is off

[    4.241890] sd 0:0:0:0: [sda] Write cache: enabled, read cache: enabled, doesn't support DPO or FUA

[    4.251771]  sda:

[    4.264635] sd 0:0:0:0: [sda] Attached SCSI disk

[    4.268947] sr 2:0:0:0: [sr0] scsi3-mmc drive: 4x/4x cd/rw xa/form2 tray

[    4.269273] cdrom: Uniform CD-ROM driver Revision: 3.20

[    4.289086] sr 2:0:0:0: Attached scsi generic sg1 type 5

[    4.392290] random: fast init done

[    4.400366] random: systemd-udevd: uninitialized urandom read (16 bytes read)

[    4.404154] random: systemd-udevd: uninitialized urandom read (16 bytes read)

[    4.404524] random: systemd-udevd: uninitialized urandom read (16 bytes read)

[    4.839472] input: ImExPS/2 Generic Explorer Mouse as /devices/platform/i8042/serio1/input/input3

[    6.797722] floppy0: no floppy controllers found

BusyBox v1.27.2 (Ubuntu 1:1.27.2-2ubuntu3.3) built-in shell (ash)

Enter 'help' for a list of built-in commands.

(initramfs) [  335.120955] clocksource: timekeeping watchdog on CPU0: Marking clocksource 'tsc' as unstable because the skew is too large:

[  335.121816] clocksource:                       'hpet' wd_now: 4fbb977d wd_last: 3fd91f2c mask: ffffffff

[  335.122506] clocksource:                       'tsc' cs_now: ef74803862 cs_last: 6aaf946b7e mask: ffffffffffffffff

[  335.123413] tsc: Marking TSC unstable due to clocksource watchdog

[  335.127797] TSC found unstable after boot, most likely due to broken BIOS. Use 'tsc=unstable'.

[  335.127938] sched_clock: Marking unstable (335110200302, 17560824)<-(335131540034, -3780156)

[  335.129147] clocksource: Switched to clocksource hpet

(initramfs)

(initramfs)

(initramfs) QEMU: Terminated

ranshal@D-P-RANSHAL-L-R

# Linux SBL → OSLoader → UEFI payload → GRUB → Linux

https://slimbootloader.github.io/how-tos/boot-ubuntu.html

SBL → OSLoader → UEFI payload → GRUB → Linux

UEFI does hw initialization

Grub does not do any hw initialization

# Build UEFI (for this flow)

### Get the edk2 source tree using Git

bash$ git clone https://github.com/tianocore/edk2.git

### Initialize submodules

bash$ git submodule update --init

### Compile build tools

bash$ cd ~/src/edk2

bash$ make -C BaseTools

bash$ . edksetup.sh

build -a IA32 -a X64 -p UefiPayloadPkg/UefiPayloadPkg.dsc -b DEBUG -t GCC5 -D BOOTLOADER=SBL

ranshal@D-P-RANSHAL-L-RF-L-RF:~/projects/SBL/edk2$ ls  Build/UefiPayloadPkgLegacy/DEBUG_GCC5/FV/ -al

total 17844

drwxrwxr-x  3 ranshal ranshal    4096 Aug 31 17:09 .

drwxrwxr-x  5 ranshal ranshal    4096 Aug 31 17:09 ..

-rw-rw-r--  1 ranshal ranshal      20 Aug 31 17:09 DXEFV.ext

-rw-rw-r--  1 ranshal ranshal 3514368 Aug 31 17:09 DXEFV.Fv

-rw-rw-r--  1 ranshal ranshal      85 Aug 31 17:09 DXEFV.Fv.map

-rw-rw-r--  1 ranshal ranshal    2986 Aug 31 17:09 DXEFV.Fv.txt

-rw-rw-r--  1 ranshal ranshal   11802 Aug 31 17:09 DXEFV.inf

drwxrwxr-x 65 ranshal ranshal    4096 Aug 31 17:09 Ffs

-rw-rw-r--  1 ranshal ranshal     464 Aug 31 17:09 GuidedSectionTools.txt

-rw-rw-r--  1 ranshal ranshal   18142 Aug 31 17:09 Guid.xref

-rw-rw-r--  1 ranshal ranshal      20 Aug 31 17:09 PLDFV.ext

-rw-rw-r--  1 ranshal ranshal 7340032 Aug 31 17:09 PLDFV.Fv

-rw-rw-r--  1 ranshal ranshal     290 Aug 31 17:09 PLDFV.Fv.map

-rw-rw-r--  1 ranshal ranshal     154 Aug 31 17:09 PLDFV.Fv.txt

-rw-rw-r--  1 ranshal ranshal    1023 Aug 31 17:09 PLDFV.inf

-rw-rw-r--  1 ranshal ranshal 7340032 Aug 31 17:09 UEFIPAYLOAD.fd

ranshal@D-P-RANSHAL-L-RF-L-RF:~/projects/SBL/edk2$

cp  Build/UefiPayloadPkgLegacy/DEBUG_GCC5/FV/UEFIPAYLOAD.fd ../slimbootloader/PayloadPkg/PayloadBins/UefiPld.fd

Note:

Using edk2
commit 6951dfe7d59d144a3a980bd7eda699db2d8554ac (HEAD, tag: edk2-stable202505)

And slimboot

commit ebc8f36d84a03a4e93261fde92b960c6ce58b12f (HEAD -> master, origin/master, origin/HEAD)

Author: Kevin Tsai <kevin.tsai@intel.com>

### Build SBL

Copy UEFI to PayloadPkg/PayloadBins/
And then:

python BuildLoader.py build qemu -p "OsLoader.efi:LLDR:Lz4;UefiPld.fd:UEFI:Lzma"

Note: when using -p and need that OsLoader will be the first bootloader than it must be explicitly mentioned in the -p string as seen here!

Shell> fs init 0:0 0 0

Init AHCI controller E00FA000

Skip reinit AHCI controller

Media(SATA) Init Success!

Find a Partition

Partition type: UNKNOWN  (1 logical partitions)

Partition Detect (device:0, hwpart:0) Success!

Init FileSystem

Detected FAT on HwDev 0 Part 0

FileSystem Detect (device:0, hwpart:0, swpart:0) Success! (FAT)

Shell> fs ls /

container.bin    52745776

Shell> fs load /container.bin 0x10000000

File was loaded at 0x10000000 with length 0x324D630!

Shell> QEMU: Terminated

# Mount your EFI image

dd if=/dev/zero of=linux-uefi.img bs=1M count=200

parted -s linux-uefi.img mklabel gpt

parted -s linux-uefi.img mkpart primary fat32 1MB 129MB

parted -s linux-uefi.img set 1 esp on

parted -s linux-uefi.img mkpart primary ext4 129MB 100%

sudo losetup -P -f linux-uefi.img

sudo mount /dev/loop24p1 efi-mount

# Remove any existing BOOTX64.EFI (the bzImage won't work)

sudo rm -f efi-mount/EFI/BOOT/BOOTX64.EFI

# Create proper directory structure

sudo mkdir -p efi-mount/boot/grub

sudo mkdir -p efi-mount/EFI

# Copy kernel and initrd

sudo cp vmlinuz-5.4.0-84-generic efi-mount/boot/vmlinuz

sudo cp initrd.img-5.4.0-84-generic efi-mount/boot/initrd.img

# Create GRUB configuration

sudo tee efi-mount/boot/grub/grub.cfg << 'EOF'

set timeout=5

set default=0

terminal_input console

terminal_output console

menuentry "Linux 5.4.0-84-generic" {

echo "Loading Linux kernel..."

linuxefi /boot/vmlinuz root=/dev/sda2 ro console=ttyS0,115200 earlyprintk=serial debug loglevel=8

echo "Loading initrd..."

initrdefi /boot/initrd.img

echo "Booting kernel..."

}

EOF

# Install GRUB EFI bootloader

sudo grub-mkimage -o efi-mount/EFI/BOOT/BOOTX64.EFI -p '/EFI/BOOT' -O x86_64-efi \

fat linux initrd normal echo configfile \

linuxefi initrdefi

# Unmount

sudo umount efi-mount

sudo losetup -d /dev/loop24

### Run Qemu

### From gemini:

That's an excellent question, and it highlights a crucial distinction in modern boot processes. The SBL → UEFI → OS Container flow is a multi-stage boot process where each component has a very specific job.

Here are the detailed steps to make this happen.

### 1. SBL's Role: The Hand-off

The Slim Bootloader's job is complete when it initializes the hardware and passes control to the UEFI payload. SBL doesn't care what the UEFI payload does or what OS it boots; its task is simply to get the system ready for the UEFI environment.

To make this happen, your SBL build must be configured to include and launch a UEFI payload. The BuildLoader.py command you mentioned does exactly that:

Bash

python BuildLoader.py build qemu -p "UefiPld.fd:UEFI:Lzma"

- UefiPld.fd: This is the UEFI payload binary, which is a full-featured UEFI firmware implementation.

- UEFI: This is the unique payload ID that SBL uses to identify this specific payload.

- Lzma: This is the compression algorithm used to store the payload in the final SBL image.

The result is a single SlimBootloader.bin image. When this image boots, the SBL will run, initialize the hardware, find the UefiPld.fd payload within its own container, load it into memory, and hand off control. At this point, the UEFI firmware takes over.

### 2. UEFI's Role: The Boot Manager

Once the UEFI firmware starts, it becomes the boot manager. Its first job is to find a bootable operating system. It does this by scanning all connected storage devices (NVMe, USB, eMMC, etc.) for a special partition called the EFI System Partition (ESP). The ESP is a dedicated partition on the disk that must be formatted with a FAT32 file system. It's the central hub for all EFI applications, including bootloaders, kernels, and system utilities.

The UEFI firmware will look for a bootloader in a standardized location on the ESP: \EFI\BOOT\bootx64.efi. If it finds this file, it will execute it.

### 3. Creating the Linux OS Container

This is where you'll create the bootable image that the UEFI firmware will load. The "container" isn't a single file for SBL, but a complete, partition-based disk image. The best way to create a minimal, easily managed Linux container is to use a UEFI stub kernel. A UEFI stub kernel is a Linux kernel compiled with the ability to act as a direct EFI executable. This means the UEFI can load and run the kernel directly, without needing an intermediate bootloader like GRUB.

Here are the command-line steps to create this container on a Linux host system.

Step 1: Create and Partition the Disk Image

First, create a blank disk image file. This will be the virtual hard drive for your OS. We'll make it 1 GB.

Bash

dd if=/dev/zero of=linux.img bs=1M count=1024

Next, partition the image. We'll create two partitions: a small EFI System Partition and a larger partition for the root file system. We'll use the parted tool for this.

Bash

parted -s linux.img mklabel gpt

parted -s linux.img mkpart primary fat32 1MB 129MB

parted -s linux.img set 1 esp on

parted -s linux.img mkpart primary ext4 129MB 100%

Step 2: Format the Partitions

Now, you need to format the partitions you created. You'll use a loop device to access the partitions inside the image file. The -P flag automatically creates device nodes for each partition.

Bash

sudo losetup -P -f --show linux.img

# This will output a device name like /dev/loop0

# You will now have device nodes like /dev/loop0p1 and /dev/loop0p2

sudo losetup -j show linux.img

Format the partitions:

Bash

sudo mkfs.fat -F32 /dev/loop22p1

sudo mkfs.ext4 /dev/loop22p2

Step 3: Mount and Populate the Partitions

Create mount points for the partitions and mount them.

Bash

mkdir -p /mnt/efi /mnt/rootfs

sudo mount /dev/loop22p1 /mnt/efi

sudo mount /dev/loop22p2 /mnt/rootfs

Now, copy your kernel and root file system.

Bash

# Populate the root file system

sudo cp -a /path/to/your/rootfs/* /mnt/rootfs/

# Create the directory for the EFI boot files

sudo mkdir -p /mnt/efi/EFI/BOOT

# Copy your UEFI stub kernel to the boot directory

sudo cp /path/to/your/bzImage /mnt/efi/EFI/BOOT/bootx64.efi

# Copy your initrd image to the same directory

sudo cp /path/to/your/initrd.img /mnt/efi/EFI/BOOT/initrd.img

sudo nano /mnt/efi/EFI/BOOT/cmdline.txt

Create the Command Line File: Inside the cmdline.txt file, paste the kernel command line arguments.

Plaintext

initrd=\\EFI\\BOOT\\initrd.img root=/dev/sda2 ro quiet splash

Step 5: Clean Up

Once all the files are copied, unmount the partitions and detach the loop device.

Bash

sudo umount /mnt/efi /mnt/rootfs

sudo losetup -d /dev/loop22

Release build with integrated OsLoader only :

python BuildLoader.py build tgl -r

python Platform/TigerlakeBoardPkg/Script/StitchLoader.py -i BVTLRxxx.bin -s Outputs/tgl/SlimBootloader.bin -o sbl_tgl_Cg_TCTL.bin

To generate container image for Tiger Lake platform:

python ./BootloaderCorePkg/Tools/GenCapsuleFirmware.py -p BIOS Outputs/tgl/SlimBootloader.bin -k keys/FirmwareUpdateTestKey_Priv_RSA2048.pem -o FwuImage.bin

| File | Description / Purpose | Contents | How it's used | Notes |
| --- | --- | --- | --- | --- |
| SlimBootloader.bin | Intermediate build output of the Slim Bootloader | Contains the built bootloader modules and optional payloads (OsLoader, UEFI) but not stitched for a specific board | Input for StitchLoader script | Not directly flashable; it’s the base binary before board-specific adjustments |
| sbl_tgl_Cg_TCTO.bin | Board-specific stitched bootloader | SlimBootloader.bin + board config (e.g., TCTO settings, memory init, payload) | Can be flashed directly to EEPROM/BIOS region for boot | Raw bootable image; may include OsLoader or UEFI payload depending on build |
| FwuImage.bin | Signed firmware update capsule | sbl_tgl_Cg_TCTO.bin wrapped in a signed container for firmware update | Used with firmware update tools (e.g., Intel FwUpdate) to safely flash EEPROM | Required if verified boot is enabled or for production updates; not a direct bootable image |

cat slimbootloader/Platform/TigerlakeBoardPkg/CfgData/CfgData_Int_Tglu_Cg_TCTO.dlt

BOOT_OPTION_CFG_DATA_0.BootFlags_0       | 0

BOOT_OPTION_CFG_DATA_0.BootDeviceType_0  | 0

BOOT_OPTION_CFG_DATA_0.HwPart_0          | 255

BOOT_OPTION_CFG_DATA_1.ImageType_1       | 0xFF

BOOT_OPTION_CFG_DATA_2.BootFlags_2       | 0

BOOT_OPTION_CFG_DATA_3.ImageType_3       | 0xFF

BOOT_OPTION_CFG_DATA_4.BootFlags_4       | 0

BOOT_OPTION_CFG_DATA_4.BootDeviceType_4  | 5

BOOT_OPTION_CFG_DATA_5.ImageType_5       | 0xFF

CAPSULE_INFO_CFG_DATA.DevType            | 0

PSD_CFG_DATA.EnablePsd                   | 0x0

==========

- $ACTION      :

page         : OS

- !expand { BOOT_OPTION_TMPL : [ 0 ,   0    ,  16 ,    5   ,   0   ,    0  ,     1 ,     1 , '/boot/sbl_os' ] }

- !expand { BOOT_OPTION_TMPL : [ 1 , 0x1F   ,  16 ,    5   ,   0   ,    0  ,     1 ,     1 , '/boot/sbl_rtcm' ] }

- !expand { BOOT_OPTION_TMPL : [ 2 ,   0    ,  16 ,    6   ,   0   ,    0  ,     1 ,     1 , '/boot/sbl_os' ] }

- !expand { BOOT_OPTION_TMPL : [ 3 , 0x1F   ,  16 ,    6   ,   0   ,    0  ,     1 ,     1 , '/boot/sbl_rtcm' ] }

- !expand { BOOT_OPTION_TMPL : [ 4 ,   0    ,  16 ,    0   ,   0   ,    0  ,     1 ,     1 , '/boot/sbl_os' ] }

- !expand { BOOT_OPTION_TMPL : [ 5 , 0x1F   ,  16 ,    0   ,   0   ,    0  ,     1 ,     1 , '/boot/sbl_rtcm' ] }

-> ext2/4 (‘1’ in table)